Plastics: Different Types

Thermoplastics: These have a “PLASTIC MEMORY”

+ve     Easily moulded into INTRICATE shapes and            economic because waste can be re-used.

+ve    Wide range of bright colours

+ve   No FINISH required

-ve    Lose rigidity and become soft when heated

- Acrylic = clear/opaque

- Rigid Polystyrene = disposable cups/cosmetic bottles

- Expanded Polystyrene = packaging

- Rigid and Flexible PVC = drain pipes/guttering hose pipes inflatable dinghies

- High and Low Density Polythene = strong and stiff used for washing up bowls/buckets and bleach bottles

Thermosetting Plastic: Heat resistant, once formed into a shape       they can not be re-shaped.

+ve Withstand higher temperatures without 						loosing rigidity and are good insulators

- Polyester Resin = comes as a thick liquid can be used in 2 ways (motorbike top boxes/boat hulls or some car bodies)

- Epoxy Resin = used to make glues (Araldite)

- Melamine Formaldehyde = strong/hard and heat resistant

- Phenol Formaldehyde = dark/hard and quite brittle (saucepan handles)

- Urea Formaldehyde = usually white in colour/tough and attractive (electrical fittings, plugs and sockets)